10    Boundary Layer Theory

10.1 A fluid with kinematic viscosity v flows in laminar stage along a flat plate with free-stream velocity V. At a distance x from the leading edge, the Reynolds number of the flow is given by R=VxV. The thickness of the boundary layer at x will be proportional to									[ESE : 2001]
(a) xR1/2		(b) xR-1/2
(c) R1/2			(d) R-1/2

10.2 Match List-I with List-II and select the correct answer using the codes given below the lists:	 [ESE : 2002]

| List-I | List-II |
| --- | --- |
| A. ∂u∂yy=0 is zero | 1. The flow is attached flow |
| B. ∂u∂yy=0 is +ve | 2. The flow is on the verge of separation |
| C. Displacement thickness | 3. ∫0δ uU1-uUdy |
| D. Momentum thickness | 4. ∫0δ 1-uUdy |

Codes:

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| (a) | 1 | 2 | 3 | 4 |
| (b) | 2 | 1 | 3 | 4 |
| (c) | 1 | 2 | 4 | 3 |
| (d) | 2 | 1 | 4 | 3 |

10.3 A thin smooth plate 1 m wide and 2 m long is towed through water at a velocity of 2 m/s. Assuming that boundary remains laminar, then drag on both sides of the plate is (kinematic viscosity =10-6 m2/s ) 																[ESE : 2003]

(a) 5.3 N	(b) 26.6 N	(c) 53 N		(d) 72.5 N

10.4 For a turbulent boundary layer (under zero pressure gradient), the velocity profile is described by the one-fifth power law. What is the ratio of displacement thickness to boundary layer thickness? 			[ESE : 2004]
(a) 1/7		(b) 1/6		(c) 1/5		(d) 1/4

10.5 In a laminar boundary layer, the velocity distribution can be assumed to be given, in usual notations, as

uv=yδ

Which one of the following is the correct expression for the displacement thickness ( δ* ) for this boundary layer?														 [ESE : 2005]
(a) δ*=δ			(b) δ*=δ/2
(c) δ*=δ/4			(d) δ*=δ/6

10.6 Which one of the following assumptions in deriving the boundary layer equation of flow past a flat plate at zero incidence is not correct? 										[ESE : 2006]
(a) Uniform flow =0
(b) Outside boundary layer velocity is v0 throughout
(c) The boundary layer thickness δ is very small compared to distance x
(d) Pressure remains constant throughout the flow both within and outside the boundary layer

10.7 Assertion (A): Boundary layer theory is applicable only in the vicinity of the leading edge of the plate.

Reason (R): Boundary layer theory is based on the assumption that its thickness is small when compared to other linear dimensions. 											[ESE : 2006]

(a) both A and R are true and R is the correct explanation of A
(b) both A and R are true but R is not a correct explanation of A
(c) A is true but R is false
(d) A is false but R is true

10.8 A flat plate with a sharp leading edge is placed along a free stream of fluid flow. Local Reynolds number at 3 cm from the leading edge is 105. What is the thickness of the boundary layer? 				[ESE : 2007]
(a) 0.47 mm			(b) 0.35 mm
(c) 0.23 mm			(d) 0.12 mm

10.9 What is the momentum thickness for the boundary layer with velocity distribution UU=yδ ? 	[ESE : 2008]
(a) δ/6		(b) δ/2		(c) 3δ/2	(d) 2δ

10.10 Assertion (A): At the point of boundary layer separation, the shear stress is zero. 		[ESE : 2011]
Reason (R): The point of separation demarcates between zones of forward and reverse flow close to the wall.
(a) both A and R are true and R is the correct explanation of A
(b) both A and R are true but R is not a correct explanation of A
(c) A is true but R is false
(d) A is false but R is true

10.11 The thickness of the laminar boundary layer over a flat plate at two different sections P and Q are 0.8 cm and 2.4 cm respectively. If the section Q is 3.6 m downstream of P, the distance of section P from the leading edge of the plate is													[ESE : 2012]
(a) 0.32 m	(b) 0.22 m	(c) 0.40 m	(d) 0.53 m

10.12 Statement-I: Boundary layer theory is applicable only in the vicinity of the leading edge of a flat plate. Statement-II: Boundary layer theory is based on the assumption that its thickness is small when compared to other linear dimensions in the flow. 										[ESE: 2012]

(a) Both Statement (I) and Statement (II) are individually true and Statement (II) is the correct explanation of Statement (I)

(b) Both Statement (I) and Statement (II) are individually true but Statement (II) is NOT the correct explanation of Statement (I)

(c) Statement (I) is true but Statement (II) is false

(d) Statement (I) is false but Statement (II) is true

10.13 The average drag coefficient for a laminar boundary layer over a flat plate was obtained as 0.018. If all other factors remain unchanged, and length of the plate is increased by 4 times its original value, this average drag coefficient would change to										[ESE: 2013]

(a) 0.0036			(b) 0.0056

(c) 0.008			(d) 0.009

10.14 The velocity distribution in a laminar flow adjacent to a solid wall is given by u=3.0 sin⁡(5πy). The viscosity of the fluid is 5 poise. What is the shear stress at a section (i) y=0.05 m; (ii) y=0.12 m ? 		[ESE : 2014]
(a) 16.7 N/m2 and 6.1 N/m2
(b) 33.4 N/m2 and Zero
(c) 16.7 N/m2 and 12.3 N/m2
(d) 16.7 N/m2 and Zero

10.15 The velocity distribution in the boundary layer is given by uU=yδ, where u is the velocity at a distance of y from the boundary and u=U at y=δ,δ being boundary layer thickness. Then the value of momentum thickness will be														[ESE : 2016]
(a) δ2		(b) δ4		(c) δ6		(d) δ8

10.16 What are the values of coefficients a and c if velocity distribution in a laminar boundary layer on a flat plat is:

f(n)=uU0=a+bn+cn2+dn3 where, n=yδ							[ESE : 2016]

(a) 12 and 1			(b) 0 and 1
(c) -12 and 0			(d) 0 and 0

Explanations Boundary Layer Theory

10.1 (b)

For Laminar boudary layer over flat plate.

δ=5xRe (Blasius solution)

10.2 (d)

- dudyy=0=0, the flow is on the verge of seperation.

- dudyy=0=+ve, the flow is attached.

- Displacement thickness =∫0δ 1-UU∞dy.

- Momentum thickness =∫0δ uu∞1-uu∞dy.

10.3(a)

Friction coefficient, Cf=1.328ReL

ReL=VLv=2×210-6=4×106 ∴Cf=13284×106=6.64×10-4

The drag force on both sides of the plate,

FD =2Cf12ρV2(BL) =2×6.64×10-4×12×1000×(2)2×1×2 =5.31 N

10.4 (b)

uu∞ =yδ1/5δ* =0δ  1-uu∞dy =0δ  1-yδ1/5dy =δ-5δ6=δ6δ*δ =16

10.5(b)

Displacement thickness

δ* =0δ  1-uvdy=0δ  1-yδdy =y-y22δ0δ=δ-δ2=δ2

10.6 (a)

Assumptions in deriving boundary layer equation

- Velocity outside boundary layer is constant throughout.

- Boundary layer thickness is very small compared to other linear dimensions.

- Pressure is constant both within and outside the boundary layer.

10.7 (d)

Statement (A) is wrong.
Since boundary layer theory is applicable not only in the vicinity of leading edge but also along the length of plate.

10.8 (a)

Since the Reynolds number is less than 5×105, the boundary layer is laminar. The thickness of the laminar boundary layer may be given by the Blasius' equation i.e.

δ=5xRex=5×3×10105⇒ δ =0.47 mm

10.9(a)

The momentum thickness is given by

θ =0δ  uU1-uUdy=0δ  yδ1-yδdy =0δ  yδ-y2δ2dy=1δ20δ  δy-y2dy =1δ2δy22-y33cδ=1δ2δ32-δ33 =δ3-26=δ6

10.10 (b)

At a point of boundary layer separation, shear stress is zero because dudy at the surface bounday is zero.

10.11 (c)

For Laminar boundary layer
Over a flat plate

δx=5Rex

δ∝x

So,      0.82.4=xx+3.6x=0.45 m

Since no any option is given.
So nearest answer is 0.4 m .

10.13 (d)

For Laminar boundary layer over a flat plate.

CD=1.328ReL=1.328ρVLμ ∴ CD∝1L ∴ CD1CD2=L2L1 ⇒0.018CD2=4L1L1 ⇒ CD2=0.009

10.14 (d)

Shear stress, τ =μ⋅∂u∂y =0.5∂∂y[3sin⁡(5πy)]

[Here μ=0.5 Pa-s]

=0.5×3×5π cos⁡(5πy)                          =23.56 cos⁡(5πy)N/m2

So, At           y =0.05 mτ =23.56cos⁡(5π×0.05) =16.7 N/m2

and At         y =0.12 mτ =23.56 cos⁡(5π×0.12) =-7.28 N/m2

Since Negative value is not acceptable.
∴ It is taken as zero.

10.15 (c)

uU =yδθ =0δ  uU1-uUdy=0δ  yδ1-yδdy =δ2-δ3=δ6

10.16 (d)

Apply boundary condition,

a=0,b=32c=0,d=-12